Building Science Control Layer Checklist

Use this checklist to verify that all four control layers are properly specified and detailed in your construction documents. A home's long-term durability depends on the continuity of these layers — gaps or breaks at transitions are where failures occur.

Project Title: ___________________________________________________________

Date: ___________________________________________________________________

Water Control Layer

| Item | Specified | Detailed | Verified | Notes |
| --- | --- | --- | --- | --- |
| Primary drainage plane / weather-resistant barrier identified |  |  |  |  |
| Roof underlayment specified |  |  |  |  |
| Flashing details at all penetrations (vents, pipes, ducts) |  |  |  |  |
| Window and door head/jamb/sill flashing sequence |  |  |  |  |
| Roof-to-wall transition flashing |  |  |  |  |
| Kickout flashings at roof/wall intersections |  |  |  |  |
| Foundation waterproofing/damp-proofing below grade |  |  |  |  |
| Deck/balcony ledger flashing and drainage |  |  |  |  |
| Material-change transition details |  |  |  |  |
| Weep drainage provisions in cladding system |  |  |  |  |

Air Control Layer

| Item | Specified | Detailed | Verified | Notes |
| --- | --- | --- | --- | --- |
| Air barrier material identified for each assembly |  |  |  |  |
| Air barrier continuity across wall-to-roof transition |  |  |  |  |
| Air barrier continuity at wall-to-foundation transition |  |  |  |  |
| Sealing strategy for electrical penetrations |  |  |  |  |
| Sealing strategy for plumbing penetrations |  |  |  |  |
| Sealing strategy for HVAC penetrations |  |  |  |  |
| Rim joist / band joist air sealing detail |  |  |  |  |
| Window and door air sealing approach |  |  |  |  |
| Top and bottom plate sealing |  |  |  |  |
| Target ACH50 documented and testing planned |  |  |  |  |

Vapor Control Layer

| Item | Specified | Detailed | Verified | Notes |
| --- | --- | --- | --- | --- |
| Vapor control strategy identified (barrier, retarder, or open) |  |  |  |  |
| Vapor profile appropriate for climate zone |  |  |  |  |
| Vapor control layer position in assembly (warm side) |  |  |  |  |
| Drying potential maintained in both directions where possible |  |  |  |  |
| Bathroom and kitchen exhaust fan specs (CFM, ducting) |  |  |  |  |
| Mechanical ventilation (ERV/HRV) vapor recovery noted |  |  |  |  |
| No double vapor barriers in any assembly |  |  |  |  |
| Below-grade vapor barrier at slab and foundation walls |  |  |  |  |

Thermal Control Layer

| Item | Specified | Detailed | Verified | Notes |
| --- | --- | --- | --- | --- |
| Insulation type and R-value specified for walls |  |  |  |  |
| Insulation type and R-value specified for roof/ceiling |  |  |  |  |
| Insulation type and R-value specified for foundation/slab |  |  |  |  |
| Continuous insulation at thermal bridges (studs, headers, corners) |  |  |  |  |
| Thermal break at slab edge / foundation-to-wall |  |  |  |  |
| Window U-values specified by orientation |  |  |  |  |
| Thermal continuity at balcony/canopy connections |  |  |  |  |
| Insulation installation quality expectations documented |  |  |  |  |

Critical Transition Points

These are where control layers are most likely to fail.

| Transition | Water | Air | Vapor | Thermal | Detail on Drawing? |
| --- | --- | --- | --- | --- | --- |
| Roof to wall |  |  |  |  |  |
| Wall to foundation/slab |  |  |  |  |  |
| Window/door openings |  |  |  |  |  |
| Deck/balcony connections |  |  |  |  |  |
| Penetrations (pipes, ducts, wires) |  |  |  |  |  |
| Material changes on exterior |  |  |  |  |  |
| Inside corners and intersections |  |  |  |  |  |
| Garage to living space separation |  |  |  |  |  |

RED FLAG:

If any transition point lacks a detailed drawing showing all four control layers, flag it with your architect before proceeding to construction documents.

Next Steps

Review the control layers at each transition point with your architect to ensure nothing is missed.

Ensure every checked item has corresponding construction document details.

Use this checklist during framing inspections and trade contractor pre-construction meetings to verify continuity.