Infrastructure Coordination Checklist

Use this checklist to ensure that lighting, audio-visual, and landscape elements are planned as infrastructure — not treated as last-minute finish selections. These systems require wires, pipes, conduit, and concrete footings that must be coordinated during design and roughed in during framing, not after drywall.

Project Title: _________________________________________________________________

Date: _________________________________________________________________

Part 1: Lighting Infrastructure

| Item | Design Phase Required | Responsible Party | Specified | Coordinated with Structure/MEP | Notes |
| --- | --- | --- | --- | --- | --- |
| Lighting concept/zones defined (ambient, task, accent) |  |  |  |  |  |
| Fixture locations marked on reflected ceiling plan |  |  |  |  |  |
| Switch and dimmer locations mapped |  |  |  |  |  |
| Circuit requirements and panel capacity confirmed |  |  |  |  |  |
| Recessed fixture clearances verified (insulation contact rating) |  |  |  |  |  |
| Under-cabinet and toe-kick lighting conduit |  |  |  |  |  |
| Exterior fixture locations and conduit runs |  |  |  |  |  |
| Landscape lighting transformer location and low-voltage runs |  |  |  |  |  |
| Dimming system / smart lighting protocol selected |  |  |  |  |  |
| Emergency/egress lighting (if required by code) |  |  |  |  |  |

Part 2: Audio-Visual & Technology Infrastructure

| Item | Design Phase Required | Responsible Party | Specified | Coordinated with Structure/MEP | Notes |
| --- | --- | --- | --- | --- | --- |
| Structured wiring plan (Cat6/fiber runs to each room) |  |  |  |  |  |
| WiFi access point locations and power/data drops |  |  |  |  |  |
| In-wall/ceiling speaker locations and wiring |  |  |  |  |  |
| Equipment closet/rack location with power, ventilation, and access |  |  |  |  |  |
| TV locations with power, data, and conduit behind wall |  |  |  |  |  |
| Outdoor speaker and camera locations with conduit |  |  |  |  |  |
| Intercom/video doorbell pre-wire |  |  |  |  |  |
| Motorized shade pockets and power (if applicable) |  |  |  |  |  |
| Security system pre-wire (sensors, keypads, panel) |  |  |  |  |  |
| Conduit for future technology runs (spare conduit paths) |  |  |  |  |  |

Part 3: Landscape Infrastructure

| Item | Design Phase Required | Responsible Party | Specified | Coordinated with Structure/MEP | Notes |
| --- | --- | --- | --- | --- | --- |
| Hardscape areas requiring footings or structural support (pergola, outdoor kitchen, fire pit) |  |  |  |  |  |
| Irrigation main line routing and zone design |  |  |  |  |  |
| Exterior drainage integration (French drains, dry wells, swales) |  |  |  |  |  |
| Pool/spa equipment pad location with utilities (electrical, gas, water) |  |  |  |  |  |
| Outdoor kitchen utilities (gas, water, electrical, drain) |  |  |  |  |  |
| Exterior lighting conduit and transformer pad |  |  |  |  |  |
| Gate/fence automation power and controls |  |  |  |  |  |
| Retaining wall engineering (if height requires) |  |  |  |  |  |
| Site grading coordination with building pad |  |  |  |  |  |
| Root barrier and tree protection during construction |  |  |  |  |  |

Conflict Check Summary

| System | Potential Conflict With | Resolution / Coordination Required | Resolved? |
| --- | --- | --- | --- |

Red Flag:

Any item marked "Not Specified" after Design Development is at risk of being missed during construction or requiring expensive change orders. Escalate unresolved items to your architect immediately.

Next Steps

1. Review each infrastructure system during the Design Development phase.

2. Assign responsibility for each item to your design team (architect, engineer, or specialist).

3. Identify and resolve all conflicts before moving to Construction Documents.

4. Use this checklist during Constructability Review to confirm all systems are ready for rough-in.