Site Prep & Foundation Inspection Checklist

Use this checklist to verify critical items during site preparation and foundation work. Everything built on top of the foundation depends on getting this phase right — and once concrete is poured, corrections are extremely expensive or impossible.

Project Title: ________________________________________________________________________________

Date: ________________________________________________________________________________

Inspector/Observer: ________________________________________________________________________________

Date of Inspection: ________________________________________________________________________________

Site Clearing & Grading

| Item | Verified (Y/N) | Date | Notes | Photo Ref |
| --- | --- | --- | --- | --- |
| Clearing limits match plans (no over-clearing) |  |  |  |  |
| Trees and vegetation marked for protection are fenced off |  |  |  |  |
| Topsoil stripped and stockpiled for final grading |  |  |  |  |
| Rough grading establishes correct building pad elevation |  |  |  |  |
| Drainage patterns directed away from building pad |  |  |  |  |
| Erosion and sediment controls installed and functional |  |  |  |  |
| Site access road stable and adequate for concrete trucks |  |  |  |  |

Excavation

| Item | Verified (Y/N) | Date | Notes | Photo Ref |
| --- | --- | --- | --- | --- |
| Excavation depth matches structural drawings |  |  |  |  |
| Soil conditions match geotechnical report assumptions |  |  |  |  |
| No unexpected water, rock, or fill material encountered |  |  |  |  |
| Excavation walls properly sloped or shored |  |  |  |  |
| Bearing capacity verified (proof roll or soil test) |  |  |  |  |
| Footing bottoms are undisturbed native soil (no loose fill) |  |  |  |  |
| Utility locations confirmed and protected |  |  |  |  |

Red Flag: If soil conditions differ from the geotech report, STOP and consult your structural engineer before proceeding. Foundation design may need modification.

Foundation Formwork & Reinforcement

| Item | Verified (Y/N) | Date | Notes | Photo Ref |
| --- | --- | --- | --- | --- |
| Footing dimensions (width, depth) match structural plans |  |  |  |  |
| Rebar size and spacing per structural drawings |  |  |  |  |
| Rebar chairs/supports maintain proper cover (3 inches typical for footings) |  |  |  |  |
| Rebar splices meet minimum lap length requirements |  |  |  |  |
| Anchor bolt locations match framing plan layout |  |  |  |  |
| Hold-down anchor locations per structural plan |  |  |  |  |
| Step footings (if any) at correct elevations |  |  |  |  |
| Form boards straight, braced, and at correct elevation |  |  |  |  |
| Keyway formed in footings for wall connection (if specified) |  |  |  |  |
| Clean-outs installed per plumbing plan |  |  |  |  |

Pre-Pour Verification

| Item | Verified (Y/N) | Date | Notes | Photo Ref |
| --- | --- | --- | --- | --- |
| All inspections passed (footing inspection by building department) |  |  |  |  |
| Plumbing under-slab rough-in complete and tested |  |  |  |  |
| Radon mitigation rough-in installed (if required) |  |  |  |  |
| Vapor barrier installed over gravel (if slab-on-grade) |  |  |  |  |
| Concrete mix design matches specifications |  |  |  |  |
| Pour schedule and sequence reviewed with contractor |  |  |  |  |
| Weather forecast acceptable for pour and cure period |  |  |  |  |
| Cure time and method specified (minimum 7 days wet cure typical) |  |  |  |  |

Foundation Waterproofing & Drainage

| Item | Verified (Y/N) | Date | Notes | Photo Ref |
| --- | --- | --- | --- | --- |
| Waterproofing or damp-proofing applied to foundation walls |  |  |  |  |
| Drainage board or dimple mat installed over waterproofing |  |  |  |  |
| Perimeter drain tile installed at footing level with proper slope |  |  |  |  |
| Drain tile connected to daylight or sump |  |  |  |  |
| Gravel backfill around drain tile (not native soil) |  |  |  |  |
| Backfill material and compaction method approved by engineer |  |  |  |  |

Issues & Resolution

| Issue | Date Found | Resolution Required | Responsible Party | Resolved Date | Verified |
| --- | --- | --- | --- | --- | --- |

Next Steps

After completing this inspection:

1. Address any items marked "Verified (Y/N): N" before proceeding

2. Document all issues and resolutions

3. Obtain building department approval if required

4. Sign and date when foundation work is approved to proceed