Framing Inspection Checklist

Use this checklist to verify structural framing before it's concealed by insulation and drywall. Framing defines your home's structural integrity, and corrections after close-in require demolition. Walk the frame with your plans in hand.

Project Title: _____________________________________________________     Date: _____________________

Inspector/Observer: ________________________     Date: _____________________

Structural Engineer Review Required?     ☐ Yes     ☐ No

Section 1: Layout Verification

| Item | Conforms (Y/N) | Location/Notes | Photo Ref |
| --- | --- | --- | --- |
| Wall locations match floor plan dimensions |  |  |  |
| Door and window rough openings match schedule (width, height, location) |  |  |  |
| Ceiling heights correct for each room/area |  |  |  |
| Floor-to-floor heights match sections |  |  |  |
| Stair opening framed to correct dimensions |  |  |  |
| Bearing walls align vertically from floor to floor |  |  |  |
| Non-bearing wall locations per plan |  |  |  |

Section 2: Structural Members

| Item | Conforms (Y/N) | Location/Notes | Photo Ref |
| --- | --- | --- | --- |
| Lumber grade and species match specifications |  |  |  |
| Beam sizes match structural drawings |  |  |  |
| Header sizes correct at all openings |  |  |  |
| Post-to-beam connections per structural details |  |  |  |
| Joist size, spacing, and direction per framing plan |  |  |  |
| Rafter or truss spacing per structural plan |  |  |  |
| Ridge beam/board sized correctly |  |  |  |
| Engineered lumber (LVL, TJI, glulam) per specs |  |  |  |

Section 3: Connection Hardware

| Item | Conforms (Y/N) | Location/Notes | Photo Ref |
| --- | --- | --- | --- |
| Hold-downs installed at locations per structural plan |  |  |  |
| Strap ties installed per plan |  |  |  |
| Joist hangers — correct size and properly nailed |  |  |  |
| Hurricane/seismic clips at roof-to-wall connections |  |  |  |
| Post bases and caps installed |  |  |  |
| Shear wall nailing pattern correct (edge spacing and field spacing) |  |  |  |
| Shear wall hold-down bolts torqued per spec |  |  |  |

Section 4: Floor System

| Item | Conforms (Y/N) | Location/Notes | Photo Ref |
| --- | --- | --- | --- |
| Subfloor material and thickness per spec |  |  |  |
| Subfloor fastening pattern (glued and screwed/nailed per code) |  |  |  |
| Blocking at bearing points and panel edges |  |  |  |
| Floor flatness acceptable (no bouncing, squeaking, or noticeable deflection) |  |  |  |
| Cantilevers framed correctly with proper bearing |  |  |  |
| Floor penetrations properly reinforced (no cut joists without engineer approval) |  |  |  |

Section 5: Roof Structure

| Item | Conforms (Y/N) | Location/Notes | Photo Ref |
| --- | --- | --- | --- |
| Roof sheathing material and thickness per spec |  |  |  |
| Sheathing nailing pattern per structural plan (H-clips if required) |  |  |  |
| Ridge connections adequate |  |  |  |
| Collar ties or rafter ties installed per plan |  |  |  |
| Ventilation openings framed (ridge vent, soffit baffles) |  |  |  |
| Roof pitch matches architectural drawings |  |  |  |
| Overhangs and fascia framing per detail |  |  |  |

Section 6: Special Conditions

| Item | Conforms (Y/N) | Location/Notes | Photo Ref |
| --- | --- | --- | --- |
| Steel-to-wood connections per structural detail |  |  |  |
| Large openings (garage, great room) properly headed and supported |  |  |  |
| Multi-story load paths continuous (loads transfer floor to floor to foundation) |  |  |  |
| Cantilevers and balconies per structural detail |  |  |  |
| Fireblocking installed at required locations (floor/ceiling penetrations, soffits) |  |  |  |
| Backing/blocking installed for future items (cabinets, TV mounts, grab bars, heavy mirrors) |  |  |  |

Section 7: Issues Found

| Item # | Location | Issue Description | Severity | Resolution Required | Resolved Date | Verified |
| --- | --- | --- | --- | --- | --- | --- |

Next Steps

• Document review with structural engineer (if required)

• Address any issues marked as "Critical" before proceeding

• Schedule follow-up inspection after corrections

• Proceed to next phase only after final approval