Building Envelope Checklist

Use this checklist to verify roofing, cladding, and weatherproofing installation. The building envelope is your home's first defense against water, air, and temperature — and the principle is simple: shed water outward, never trap moisture. Every detail matters because a single gap in the envelope can cause years of hidden damage.

Project Title: _____________________________________________________     Date: _____________________

Inspector/Observer: ________________________     Date: _____________________

Section 1: Roof System

| Item | Verified (Y/N) | Date | Notes | Photo Ref |
| --- | --- | --- | --- | --- |
| Roof sheathing complete with no gaps or damage |  |  |  |  |
| Underlayment installed per spec (ice and water shield at eaves, valleys, penetrations) |  |  |  |  |
| Underlayment lapped correctly (upper over lower, water sheds downhill) |  |  |  |  |
| Roofing material matches specifications |  |  |  |  |
| Flashing at all roof penetrations (vents, pipes, chimneys) |  |  |  |  |
| Valley flashing installed correctly |  |  |  |  |
| Ridge cap/vent installed |  |  |  |  |
| Drip edge at eaves and rakes |  |  |  |  |
| Gutter and downspout system roughed in or planned |  |  |  |  |
| Roof-to-wall transition flashing sealed and integrated with wall WRB |  |  |  |  |

Section 2: Wall Weather-Resistant Barrier (WRB)

| Item | Verified (Y/N) | Date | Notes | Photo Ref |
| --- | --- | --- | --- | --- |
| WRB material matches specifications (housewrap, fluid-applied, etc.) |  |  |  |  |
| WRB installed shingle-style (upper courses lap over lower) |  |  |  |  |
| All seams taped or sealed per manufacturer requirements |  |  |  |  |
| WRB integrated with window/door flashing (correct lapping sequence) |  |  |  |  |
| WRB extends to foundation or terminates at proper detail |  |  |  |  |
| No tears, punctures, or unsealed fastener penetrations |  |  |  |  |
| Proper overlap at corners (minimum per spec) |  |  |  |  |

Section 3: Window and Door Integration

| Item | Verified (Y/N) | Date | Notes | Photo Ref |
| --- | --- | --- | --- | --- |
| Sill pan flashing installed at all openings before window/door installation |  |  |  |  |
| Head flashing extends beyond jamb and laps over WRB |  |  |  |  |
| Jamb flashing integrated with sill pan |  |  |  |  |
| WRB laps over head flashing (water sheds outward at every layer) |  |  |  |  |
| Sealant applied per window manufacturer and flashing manufacturer requirements |  |  |  |  |
| No reverse laps (lower material should never lap over upper) |  |  |  |  |

Section 4: Exterior Cladding

| Item | Verified (Y/N) | Date | Notes | Photo Ref |
| --- | --- | --- | --- | --- |
| Cladding material matches specifications |  |  |  |  |
| Rainscreen gap/drainage plane behind cladding (if specified) |  |  |  |  |
| Weep screeds or drainage provisions at base of wall |  |  |  |  |
| Kickout flashings at roof-to-wall intersections |  |  |  |  |
| Control joints or expansion joints at required intervals |  |  |  |  |
| Cladding fastening per manufacturer requirements |  |  |  |  |
| Material-change transitions properly flashed and sealed |  |  |  |  |
| Minimum clearance from grade maintained (6-8 inches typical) |  |  |  |  |

Section 5: Below-Grade Envelope

| Item | Verified (Y/N) | Date | Notes | Photo Ref |
| --- | --- | --- | --- | --- |
| Foundation waterproofing/damp-proofing intact (no damage from backfill) |  |  |  |  |
| Drainage board or dimple mat in place |  |  |  |  |
| Perimeter drain functioning (verified before backfill) |  |  |  |  |
| Backfill material and compaction per specification |  |  |  |  |
| Grade slopes away from foundation (minimum 6 inches in first 10 feet) |  |  |  |  |

Section 6: Control Layer Continuity Check

| Transition Point | Water Layer Continuous? | Air Layer Continuous? | Notes |
| --- | --- | --- | --- |
| Roof to wall |  |  |  |
| Wall to foundation |  |  |  |
| Window/door openings |  |  |  |
| Deck or balcony attachment |  |  |  |
| Penetrations (pipes, ducts, vents, wires) |  |  |  |
| Material changes on exterior |  |  |  |
| Garage to living space |  |  |  |

Red Flag: Any "No" answer in this section means there is a gap in the envelope that will allow water or air intrusion. Resolve before proceeding.

Section 7: Issues Found

| Item # | Location | Issue | Severity | Resolution | Date Fixed | Verified |
| --- | --- | --- | --- | --- | --- | --- |

Next Steps

• Document review with structural engineer (if required)

• Address any issues marked as "Critical" before proceeding

• Schedule follow-up inspection after corrections

• Proceed to next phase only after final approval