Window & Door Installation Checklist

Use this checklist to verify every window and door installation for quality, weather-tightness, and proper operation. Windows and doors are the most complex penetrations in your building envelope — each one is a potential failure point for water, air, and thermal performance.

| Project Title: _____________________________ | Date: _____________________________ |
| --- | --- |

| Inspector/Observer: _______________________ | Date: _____________________________ |
| --- | --- |

## Section 1: Delivery Verification

| Item | Verified (Y/N) | Notes |
| --- | --- | --- |
| All units received match order (count, sizes, configurations) |  |  |
| Manufacturer and product line match specifications |  |  |
| Frame material and color match selections |  |  |
| Glass type matches spec (Low-E coating, double/triple, gas fill) |  |  |
| Hardware finish and style match selections |  |  |
| No shipping damage (check frames, glass, hardware, weatherstripping) |  |  |
| Units stored properly on site (upright, protected from weather and construction debris) |  |  |

## Section 2: Window Installation — Per Unit Checklist

Window Location: ____________   Size: ____________   Manufacturer: ____________

| Item | Verified (Y/N) | Notes |
| --- | --- | --- |
| Rough opening dimensions correct (shimming space adequate, not excessive) |  |  |
| Sill pan flashing installed before window |  |  |
| Unit centered and shimmed in opening |  |  |
| Unit is level and plumb |  |  |
| Unit is square (diagonal measurements equal) |  |  |
| Fasteners at correct locations and spacing per manufacturer |  |  |
| Shims at fastener locations (unit not racked by fastening) |  |  |
| Gap between frame and rough opening sealed with appropriate material (backer rod + sealant or low-expansion foam) |  |  |
| Interior air seal complete |  |  |
| Exterior flashing integrated with WRB (correct lapping sequence) |  |  |
| Sash operates smoothly (opens, closes, locks without force) |  |  |
| Lock engages fully |  |  |
| Weatherstripping intact and making full contact |  |  |
| No visible light gaps between sash and frame |  |  |
| Interior trim/casing installed and sealed (if applicable at this stage) |  |  |

## Section 3: Exterior Door Installation

Door Location: ____________   Type: ____________   Manufacturer: ____________

| Item | Verified (Y/N) | Notes |
| --- | --- | --- |
| Rough opening dimensions correct |  |  |
| Threshold/sill pan properly installed and flashed |  |  |
| Door frame plumb and square |  |  |
| Door swings freely without dragging or binding |  |  |
| Deadbolt and latch engage smoothly |  |  |
| Weatherstripping makes full contact on all four sides |  |  |
| Threshold seal compresses against door bottom (no daylight visible) |  |  |
| Glass panels (if any) match specifications |  |  |
| Multipoint lock operates correctly (if applicable) |  |  |
| Exterior flashing and sealant integrated with WRB |  |  |
| Hardware finish matches schedule |  |  |

## Section 4: Sliding/Folding Door Systems (if applicable)

| Item | Verified (Y/N) | Notes |
| --- | --- | --- |
| Track level and properly anchored |  |  |
| Panels slide/fold smoothly without resistance |  |  |
| Panels align when closed (no gaps) |  |  |
| Locking mechanism engages fully |  |  |
| Drainage weeps at track/sill clear and functional |  |  |
| Structural header adequate for opening span |  |  |
| Threshold transition to floor material planned |  |  |

## Section 5: Performance Spot-Check

| Item | Verified (Y/N) | Notes |
| --- | --- | --- |
| Selected windows tested for air infiltration (hold tissue at edges — should not flutter) |  |  |
| No condensation between glass panes (indicates seal failure) |  |  |
| Operable windows hold their position when opened (don't slide shut) |  |  |
| Screen frames fit properly and are undamaged |  |  |

## Section 6: Issues Found

| Item # | Location | Issue | Severity | Resolution | Date Fixed | Verified |
| --- | --- | --- | --- | --- | --- | --- |

## Next Steps

After completing window and door installation and resolving any issues, proceed with exterior finishing, weatherproofing, and interior trim work.