MEP Rough-In Inspection Checklist

Use this checklist to verify plumbing, electrical, and HVAC rough-in work before walls are closed. MEP systems involve three or more trades working in the same wall and ceiling cavities — coordination failures here become expensive conflicts later. Verify each trade independently, then check for conflicts between them.

Project Title: _________________________     Date: _________________________     Inspector/Observer: _________________________

1. Plumbing Rough-In

| Item | Verified (Y/N) | Notes | Photo Ref |
| --- | --- | --- | --- |
| Supply pipe material and sizing match plans |  |  |  |
| Hot water pipe on left side when facing fixture (standard) |  |  |  |
| Drain pipe sizing correct per code |  |  |  |
| Drain pipe slope correct (1/4 inch per foot typical) |  |  |  |
| Vent pipes properly connected and routed to exterior |  |  |  |
| Vent terminations per code (height above roof, distance from windows) |  |  |  |
| Water heater location matches plan with required clearances |  |  |  |
| Fixture rough-in dimensions match selected fixtures (verify heights, offsets) |  |  |  |
| Shut-off valve locations accessible |  |  |  |
| Hose bibb locations per plan |  |  |  |
| Gas piping sized and routed correctly (if applicable) |  |  |  |
| Pressure test completed and passed (test pressure: ___ PSI for ___ minutes) |  |  |  |
| No leaks detected at any joint or connection |  |  |  |

2. Electrical Rough-In

| Item | Verified (Y/N) | Notes | Photo Ref |
| --- | --- | --- | --- |
| Panel location matches plan with required working clearance (36 inches front, 30 inches wide) |  |  |  |
| Panel capacity adequate for planned load (total amps: ___) |  |  |  |
| Wire gauge correct for each circuit type (verify at panel and at devices) |  |  |  |
| Dedicated circuits installed for: range, oven, dishwasher, disposal, microwave, refrigerator, washer, dryer, HVAC, water heater, garage door opener |  |  |  |
| GFCI protection in all required locations (kitchen, bath, garage, exterior, laundry, unfinished basement) |  |  |  |
| AFCI protection in all required locations (bedrooms, and per local code) |  |  |  |
| Smoke detector locations per code (every bedroom, outside sleeping areas, each level) |  |  |  |
| Carbon monoxide detector locations per code (near sleeping areas, near fuel-burning appliances) |  |  |  |
| Outlet heights and spacing per plan and code |  |  |  |
| Switch heights consistent (48 inches typical unless ADA) |  |  |  |
| Three-way and four-way switch wiring correct |  |  |  |
| Exterior outlet locations per plan with weather-rated boxes |  |  |  |
| Recessed lighting cans are IC-rated where insulation contact occurs |  |  |  |

3. HVAC Rough-In

| Item | Verified (Y/N) | Notes | Photo Ref |
| --- | --- | --- | --- |
| Equipment location matches plan (furnace, air handler, condenser) |  |  |  |
| Ductwork material matches specifications |  |  |  |
| Duct sizing per HVAC design (Manual D) |  |  |  |
| Supply register locations match HVAC plan |  |  |  |
| Return air locations and sizing adequate |  |  |  |
| Duct joints sealed with mastic (not tape alone) |  |  |  |
| Duct insulation installed where specified (unconditioned spaces) |  |  |  |
| Condensate drain line routed to approved location |  |  |  |
| Refrigerant line routing clear and protected |  |  |  |
| Thermostat wiring at correct locations |  |  |  |
| Exhaust fan ducting to exterior (bath fans, kitchen hood) |  |  |  |
| Combustion air provisions adequate (if applicable) |  |  |  |
| Fresh air intake (ERV/HRV) ducted per plan |  |  |  |

4. Smart Home & Low-Voltage

| Item | Verified (Y/N) | Notes | Photo Ref |
| --- | --- | --- | --- |
| Cat6/Cat6a network cables pulled to each planned location |  |  |  |
| WiFi access point locations have power and data drops |  |  |  |
| In-wall/ceiling speaker wiring pulled to locations |  |  |  |
| Equipment closet/rack has adequate power outlets, data connections, and ventilation |  |  |  |
| TV wall locations have power, data, and conduit or media plate |  |  |  |
| Security system wiring (sensors, keypads, cameras) per plan |  |  |  |
| Doorbell/intercom wiring per plan |  |  |  |
| Motorized shade power at window locations (if applicable) |  |  |  |
| Spare conduit runs for future technology |  |  |  |

5. Inter-Trade Coordination Check

| Check | OK (Y/N) | Issue Description | Resolution |
| --- | --- | --- | --- |
| No plumbing pipes notched into structural members beyond allowable limits |  |  |  |
| No electrical wires compressed by plumbing or HVAC |  |  |  |
| HVAC ducts not conflicting with structural members or plumbing |  |  |  |
| All penetrations through fire-rated assemblies properly firestopped |  |  |  |
| Adequate space for insulation around all MEP runs |  |  |  |
| Access panels planned for concealed valves, cleanouts, and equipment |  |  |  |

6. Issues Found

| Item # | Trade | Location | Issue | Resolution | Resolved | Verified |
| --- | --- | --- | --- | --- | --- | --- |

Next Steps

Ensure all items on this checklist have been verified and any issues resolved. Schedule follow-up inspections as needed. Coordinate with all trades to confirm understanding of requirements.