Drywall & Interior Surface Checklist

Use this checklist to verify drywall and interior surface quality at two stages: before hanging (all rough-in work approved and blocking installed) and after finishing (smooth, flat surfaces ready for paint). Good drywall work is invisible — you only notice it when it's bad.

Project Title: _________________________     Date: _________________________     Inspector/Observer: _________________________

Stage 1: Pre-Hanging Verification

| Item | Verified (Y/N) | Notes |
| --- | --- | --- |
| All rough-in inspections passed (electrical, plumbing, HVAC, insulation) |  |  |
| Pre-drywall checkpoint completed and signed off |  |  |
| Backing/blocking installed at all required locations (cabinets, TV mounts, grab bars, heavy shelving, towel bars, mirrors) |  |  |
| Shower/tub surrounds framed and backed for tile or panel installation |  |  |
| Vapor barrier installed where specified (check before it's covered) |  |  |
| Electrical box depths set for finish wall thickness |  |  |
| Recessed light cans installed at correct depth |  |  |
| Medicine cabinet and niche framing at correct dimensions |  |  |
| HVAC register boot locations correct and accessible |  |  |

Stage 2: Drywall Hanging

| Item | Verified (Y/N) | Notes |
| --- | --- | --- |
| Correct drywall type by location: standard where specified |  |  |
| 5/8 inch Type X (fire-rated) at garage ceiling, furnace room, and between-unit separations |  |  |
| Moisture-resistant (green board or equivalent) in non-shower wet areas |  |  |
| Cement board or equivalent at tile backer areas (shower, tub surround) |  |  |
| No drywall sitting directly on concrete or wet surfaces (gap at floor) |  |  |
| Fastener spacing per code (screws every 12 inches on ceilings, 16 inches on walls typical) |  |  |
| No missed or popped fasteners |  |  |
| Joints staggered (not aligned at corners or openings) |  |  |
| Cut-outs clean around electrical boxes, switches, and outlets (no large gaps) |  |  |
| Sound insulation installed at designated walls before closing (media room, bedrooms, bathrooms) |  |  |

Stage 3: Drywall Finishing — Level of Finish

Drywall finish levels range from Level 1 (fire tape only) to Level 5 (skim coat). Most residential work requires Level 4 (tape, two coats mud, sanded smooth). Areas that will receive glossy paint, critical lighting, or texture may need Level 5.
Specified finish level: ___ (typical residential: Level 4)

| Item | Verified (Y/N) | Notes |
| --- | --- | --- |
| All joints taped and coated to specified level |  |  |
| Joints are smooth and feathered — no visible ridges, bumps, or tool marks |  |  |
| Fastener heads fully covered and smooth |  |  |
| Inside corners straight and clean |  |  |
| Outside corners protected with corner bead, straight and consistent |  |  |
| No cracks, bubbles, or delamination in joint compound |  |  |
| Sanding complete with no visible scratch patterns |  |  |
| Surface ready for primer (clean, dust-free, smooth) |  |  |
| Any specified texture applied consistently |  |  |

Stage 4: Quality Issues to Watch For

| Issue | Present? (Y/N) | Location | Action Required |
| --- | --- | --- | --- |
| Nail pops (fasteners pushing through) |  |  |  |
| Joint cracking (especially at corners and ceiling joints) |  |  |  |
| Uneven surfaces (check with raking light or straight edge) |  |  |  |
| Bulging at joints (over-mudded) |  |  |  |
| Rough or unsanded areas |  |  |  |
| Gaps at floor, ceiling, or corners |  |  |  |
| Damage from subsequent trades (common — plan for touch-up) |  |  |  |

Note on Drywall Damage

Expect drywall damage from subsequent trades (trim carpenters, cabinet installers, painters will inevitably ding fresh drywall). Include a drywall touch-up visit in the schedule before final paint.

Issues Found

| Item # | Location | Issue | Resolution | Fixed | Verified |
| --- | --- | --- | --- | --- | --- |

Next Steps

Ensure all items on this checklist have been verified and any issues resolved. Schedule follow-up inspections as needed. Coordinate with all trades to confirm understanding of requirements.